Scooptram XYWJ-8.5

Technical specification

The Diesel Scooptram is capacity of 16 tons. It’s driven by Mercedes Benz Diesel Engine OM502LA, improves productivity and underground tunnel, and reduces the pollution efficiently.

Characteristics

Structure

1. Z-bar device

2. Lockup for boom & steering

3.V shape bucket

Safe Operation

1.SAHR brake system

2.ROPS/FOPS Certificate

Comfortable Operation

1. Ergonomics Canopy with generous operation room,

2. Enclosed canopy with air conditioner

3. Noise ≤85db

4. Adjustable operator’s seat

Efficient Production

- Matched Powertrain

- Digital Monitor & diagnose

- Lube Layout to maintain

Main Technical Parameter

| # Bucket motion times |  |
| --- | --- |
| Raising Time: | 7.3s |
| Dropping Time: | 4.0s |
| Tipping Time: | 3.0s |
| Max. Lifting Height: | 6800mm |
| Max. Discharging Height: | 2750mm |
| Bucket pivot at Max. lifting: | 4600mm |
| Min. Discharging Distance: | 1850mm |
| Discharging Angle: | 40° |
| Wheel Base: | 3860mm |
| Leaving Angle: | 14° |
| Rear Axle Oscillation Angle: | ±8° |
| Min. Off-ground Clearance: | 500mm |
| Max. gradeability: | 25% |
| Max. Steering Angle: | ± 42 ° |

| Dimension (tramming): | 11280mm×3100mm×2800mm |
| --- | --- |
| Bucket Capacity(SAE): | 8m³ |
| Weights |  |
| Operating Weight: | 47000 kg |
| RatedLoading: Capacity: | 16000 kg |
| Axle weights without load |  |
| Front Axle: | 20500 kg |
| Rear Axle: | 26500 kg |
| Axle weights with load: |  |
| Front Axle: | 44500 kg |
| Rear Axle: | 19500 kg |
| Driving Speeds Forward and Reverse |  |
| First Gear: | 0~6.8 km/h |
| Second Gear: | 0~11.4 km/h |
| Third Gear: | 0~19.0 km/h |
| Forth Gear: | 0~27.1 km/h |
| Min. Steering Radius |  |
| Outer / Inner: | 7350 mm/3460 mm |

Main Configuration

| Hydraulic System |  |
| --- | --- |
| Components: | Servicing system, steering system, braking system, fill oil system |
| Features: | The hydraulic system use PARKER’s hydraulic pumps and valves. The braking system use MICO’s hydraulic components. Using the method of dual pumps confluence, in order to reduce the energy loss. Rated Pressure of service hydraulics: 28Mpa |
| Braking System |  |
| Components: | Service/Parking/Emergency brake |
| Features: | These brakes are spring applied, hydraulically released type (SAHR)，with fully enclosed, multiple wet discs that are forced-cooled at each wheel end. Equiped with a hand pump to release the brake in emergency |

| Diesel Engine |  |
| --- | --- |
| Manufacturer: | Mercedes Benz |
| Model: | OM460LA |
| Rated Power: | 315KW@1800rpm |
| Features: | Water Cooled, Turbo Charged Catalytic fume purifier |
| Hydraulic Torque Converter |  |
| Manufacture: | DANA |
| Model: | C9000 Series |
| Power Shift Transmission |  |
| Manufacture: | DANA |
| Model: | 6000 Series |
| Features: | F4/R4 Electronic control Modulation |
| Drive Axle |  |
| Manufacture: | KESSLER |
| Model: | D106 |
| Tyres |  |
| Manufacture: | China Tyres Company |
| Model: | 29.5x29 L5S |

| Electric System |  |
| --- | --- |
| Alternator: | 28V 100A |
| Batteries: | 12VX2 195Ah |
| Lights: | Front: 5 pcs Rear: 4 pcs |
| Reverse Alarm | YES |
| Sensors: | Fuel/Oil level sensors, Sensors of service, steering, brake system, Limit Sensor |
| Alarm: | - Red/green indicators - Rotation Warning Lamp - Low oil level warning/Oil pressure warning - Engine failure Warning |
| Monitor Screen: | IFM CR1083 7” LED |
| Controller: | IFM CR2532 CPU 50MHz |
| Cab |  |
| Features: | - ROPS/FOPS approved - Side Seated operator for bi-directional operation - Adjustable Spring Cushioned Seat with Three Points Contact Seat Belt - Cab is removable independently |

Fuel & Lubricant Specifications

Diesel Fuel: ASTM D975 Grade 2D

Engine Oil: SAE 15W-40

Hydraulic Oil: ISO VG 46

Transmission Fluid: SAE 30

Optional equipment

Lincoln auto lube with timer

Ansul Checkfire, autofire suppression

Fully Enclosed Cab with Air Conditioner

Customized Bucket Capacity

SAFETY

- Neutral Brake - Brake automatically apply when engine stoped - Automatically open cabin door braking - Brake automatically apply with low hydraulic oil level or low fuel level - Brake automatically applies with low accumulator pressure. - Reflective bands on front, rear and both sides of the vehicle. - Wheel stop blocks