Scooptram XYWJ-8.5

Technical specification

The Electric Scooptram is capacity of 17 tons. It’s driven by electric motor, improves productivity and underground tunnel, and reduces the pollution efficiently.

Characteristics

Structure

1. Boom with Z-bar Linkage

2. Horizontal Cable roller

3. Lockup for boom & steering

4.Cable length 350m

Safe Operation

1.SAHR brake system

2.ROPS/FOPS Certificate

Comfortable Operation

1. Ergonomics Canopy with generous operation room,

2. Enclosed canopy with air conditioner

3. Noise ≤85db

4. Adjustable operator’s seat

Efficient Production

- Matched Powertrain

- Digital Monitor & diagnose

- Lube Layout to maintain

Main Technical Parameter

| # Bucket motion times |  |
| --- | --- |
| Raising Time: | 7.3s |
| Dropping Time: | 4.0s |
| Tipping Time: | 3.0s |
| Max. Lifting Height: | 6800mm |
| Max. Discharging Height: | 2750mm |
| Bucket pivot at Max. lifting: | 4600mm |
| Min. Discharging Distance: | 1850mm |
| Discharging Angle: | 40° |
| Wheel Base: | 3860mm |
| Leaving Angle: | 14° |
| Rear Axle Oscillation Angle: | ±8° |
| Min. Off-ground Clearance: | 500mm |
| Max. gradeability: | 25% |
| Max. Steering Angle: | ± 42 ° |

| Dimension (tramming): | 11750mm×3100mm×2800mm |
| --- | --- |
| Bucket Capacity(SAE): | 7m³ |
| Weights |  |
| Operating Weight: | 48000 kg |
| Rated Loading: Capacity: | 17000 kg |
| Axle weights without load |  |
| Front Axle: | 21000 kg |
| Rear Axle: | 27000 kg |
| Axle weights with load: |  |
| Front Axle: | 46000 kg |
| Rear Axle: | 19000 kg |
| Driving Speeds Forward and Reverse |  |
| First Gear: | 0~3.5 km/h |
| Second Gear: | 0~6.1 km/h |
| Third Gear: | 0~10.3 km/h |
| Forth Gear: | 0~17.7 km/h |
| Min. Steering Radius |  |
| Outer / Inner: | 7350 mm/3460 mm |

Main Configuration

| Hydraulic System |  |
| --- | --- |
| Components: | Servicing system, steering system, braking system, fill oil system |
| Features: | Service and steering hydraulic systems are equipped with PARKER Hydraulic Pumps and Valves, and brake hydraulic system equipped with Mico hydraulic components. The oil flow from steering hydraulic pump is directed to bucket hydraulics when steering is not used. Rated Pressure of service hydraulics: 28Mpa |
| Braking System |  |
| Components: | Service/Parking/Emergency brake |
| Features: | These brakes are spring applied, hydraulically released type (SAHR)，with fully enclosed, multiple wet discs that are forced-cooled at each wheel end. Equiped with a hand pump to release the brake in emergency |

| Electric Motor |  |
| --- | --- |
| Type: | Three-phase, squirrel-cage motor |
| Model: | HM2-315L2-4 |
| Power: | 160KW@1480rpm |
| Operating Voltage: | 1000V |
| Frequency: | 50 Hz |
| Hydraulic Torque Converter |  |
| Manufacture: | DANA |
| Model: | C9000 Series |
| Power Shift Transmission |  |
| Manufacture: | DANA |
| Model: | 6000 Series |
| Features: | F4/R4 Electronic control Modulation |
| Drive Axle |  |
| Manufacture: | KESSLER |
| Model: | D106 |
| Tyres |  |
| Manufacture: | China Tyres Company |
| Model: | 29.5x29 L5S |

| Electric System |  |
| --- | --- |
| Control Voltage | 24V |
| Lights: | Front: 5 pcs Rear: 4 pcs |
| Reverse Alarm | YES |
| Sensors: | Fuel/Oil level sensors, Sensors of service, steering, brake system, Limit Sensor |
| Alarm: | - Red/green indicators - Rotation Warning Lamp - Low oil level warning/Oil pressure warning - Engine failure Warning |
| Monitor Screen: | IFM CR1083 7” LED |
| Controller: | IFM CR2532 CPU 50MHz |
| Cab |  |
| Features: | - ROPS/FOPS approved - Side Seated operator for bi-directional operation - Adjustable Spring Cushioned Seat with Three Points Belt - Cab is removable independently |

Fuel & Lubricant Specifications

Hydraulic Oil: ISO VG 46

Transmission Fluid: SAE 30

Optional equipment

Lincoln auto lube with timer

Ansul Checkfire, autofire suppression

Fully Enclosed Cab with Air Conditioner

Customized Bucket Capacity

SAFETY

- Brake automatically applied in Neutral - Brake automatically apply when engine shutdown - Door Interlock - Brake automatically apply with low hydraulic oil level or low fuel level - Brake automatically applies with low accumulator pressure. - Reflective bands on front, rear and both sides of the vehicle. - Wheel stop blocks - Gear Selector automatically to Neutral when machine fails in axle or hydraulic oil level low.