Scooptram XYWJD-4

Technical specification

The Electric Scooptram is capacity of 10 tons. It’s driven by electric motor, improves productivity and underground tunnel, and reduces the pollution efficiently.

Characteristics

Structure

1. Z-bar device

2. Lockup for boom & steering

3.V shape bucket

Safe Operation

1.SAHR brake system

2.ROPS/FOPS Certificate

Comfortable Operation

1. Ergonomics Canopy with generous operation room,

2. Enclosed canopy with air conditioner

3. Noise ≤85db

4. Adjustable operator’s seat

Efficient Production

- Matched Powertrain

- Digital Monitor & diagnose

- Lube Layout to maintain

Main Technical Parameter

| 主要技术参数 # Bucket motion times 工作时间 |  |
| --- | --- |
| Raising Time: 举臂时间: | 9.6s |
| Lowering Time: 落臂时间: | 6.6s |
| Tipping Time: 翻斗时间: | 4.0s |
| Max. Lifting Height: 最大举升时的整机高度: | 5088mm |
| Max. Discharging Height: 最大卸载高度: | 1805mm |
| Bucket pivot at Max. lifting: 最大举升时的铰销高度: | 3410mm |
| Min. Discharging Distance: 最小卸载距离： | 1710mm |
| Discharging Angle: 最大卸载角度： | 42° |
| Wheel Base: 轴距: | 3600mm |
| Leaving Angle: 离去角 | 15° |
| Rear Axle Oscillation Angle: 后桥摆动角： | ±8° |
| Min. Off-ground Clearance: 最小离地间隙： | 358mm |
| Max. gradeability: 爬坡能力： | 25% |
| Max. Steering Angle: 最大转向角： | ± 42 ° |

| Dimension (tramming): 外形尺寸（运输状态） | 9870mm×2500mm×2500mm |
| --- | --- |
| Bucket Capacity(SAE): 铲斗容量(SAE) | 4m³ |
| Weights 重量 |  |
| Operating Weight: 操作重量: | 27800 kg |
| Rated Loading Capacity: 额定载重量: | 10000 kg |
| Axle weights without load 空载 |  |
| Front Axle: 前桥载荷: | 12800kg |
| Rear Axle: 后桥载荷: | 15000kg |
| Axle weights with load 满载 |  |
| Front Axle: 前桥载荷: | 26000kg |
| Rear Axle: 后桥载荷: | 11800kg |
| Driving Speeds Forward and Reverse 运行能力 |  |
| First Gear: 1档: | 0~2.8km/h |
| Second Gear: 2档: | 0~5.8km/h |
| Third Gear: 3档: | 0~9.9km/h |
| Forth Gear: 4档: | 0~16.7km/h |
| Min. Steering Radius 最小转向半径 |  |
| Outer / Inner: 外侧 / 内侧: | 6600 mm/3560mm |

Main Configuration

主要配置

| Electric Motor 电动机 |  |
| --- | --- |
| Type: 类型: | Three-phase, squirrel-cage motor 三相鼠笼型 |
| Model: 型号: | YX3-315M-4 |
| Power: 功率: | 132KW@1480rpm |
| Operating Voltage 工作电压 | 1000V(660V customized) |
| Frequency: 频率: | 50 Hz |
| Hydraulic Torque Converter 液力变矩器 |  |
| Manufacture: 制造厂: | DANA 美国德纳 |
| Model: 型号: | C5000 Series |
| Power Shift Transmission 变速箱 |  |
| Manufacture 制造厂: | DANA 美国德纳 |
| Model: 型号: | R36000 Series |
| Features: 特 点: | 前/后四档 电子控制 F4/R4 Electronic control |
| Drive Axle 驱动桥 |  |
| Manufacture: 制造厂: | KESSLER 德国凯斯勒 |
| Model: 型号: | D101 |
| Tyres 轮胎 |  |
| Manufacture: 制造厂: | China Tyres Company 中国轮胎 |
| Model: 型号: | 18.00-25 |

| Hydraulic System 液压系统： |  |
| --- | --- |
| Components: 组成: | Servicing system, steering system, braking system, charge system, Roller System 工作系统、转向系统、 制动系统 、补油系统、卷缆系统 |
| Features: 特 点: | Service and steering hydraulic systems are equipped with PARKER Hydraulic Pumps and Valves, and brake hydraulic system equipped with Mico hydraulic components. The oil flow from steering hydraulic pump is directed to bucket hydraulics when steering is not used. Rated Pressure of service hydraulics: 19Mpa 液压泵和阀采用派克品牌，制动系统液压元件采用米肯品牌。 采用双泵合流，减少能量损失。工作系统压力19Mpa |
| Braking System 制动系统 |  |
| Components: 组 成: | Service/Parking/Emergency brake 工作制动、停车制动和紧急制动 |
| Features: 特 点: | These brakes are spring applied, hydraulically released type (SAHR)，with fully enclosed, multiple wet discs that are forced-cooled at each wheel end. 工作制动、停车制动为弹簧制动，液压释放. 制动器为全封闭液压湿式多盘制动器 |

| Electric System 电器系统 |  |  |
| --- | --- | --- |
| Lights: 工作灯: |  | Front: 5 pcs Rear: 3 pcs |
| Sensors: 传感器: |  | Oil level sensors, Sensors of service, steering, brake system, Limit Sensor 液压油箱油位传感器，工作、转向、制动系统压力传感器，限位传感器 |
| Alarm: 报警提示: |  | Red/green indicators 红绿指示灯 Rotation Warning Lamp 旋转报警灯 Low oil level warning/Oil pressure warning 低油位报警/油压报警 |
| Monitor Screen: 显示器 |  | IFM CR1083 7” LED Display |
| Controller: 控制器 |  | IFM CR2532 50MHz |
| Control Voltage: 控制电压: |  | 24V |
| Cab: 驾驶室： |  |  |
| Features: 特点: | - ROPS/FOPS approved 符合ROPS/FOPS标准 - Side Seated operator for bi-directional operation 驾驶员侧坐以便双向操作 - Adjustable Spring Cushioned Seat with Three Points Belt 可调弹簧减震座椅配安全带 |  |

Optional equipment 可选配置

Lincoln auto lube with timer 林肯润滑系统

Ansul Checkfire, autofire suppression 安索集中灭火系统

Fully Enclosed Cab with Air Conditioner 全封闭空调驾驶室

Customized Bucket Capacity 可定制其他斗容