Scooptram XYWJY-4

Technical specification

The 4 m³remote Scooptram is capacity of 10 tons. It’s driven by BF6M1015C Diesel Engine. It features powerful and high productivity. It can be shifted between remote and manual operation, and the remote distance is 180m, especially for underground dangerous circumstance.

Characteristics

Structure

1. Z-bar device

2. Lockup for boom & steering

3. V shape bucket

安全的工作环境

1.SAHR brake system

2.ROPS/FOPS Certificate

3.Remote Distance 180m

遥控功能齐全：

1. 7-way PWM signal, it can remote the operations of turning left/right, Lifting/lowering, bucket tipping /backing and accelerator.

2. 4-way digital signal input, change of remote/manual, run testing of brake, neutral and warning.

高效生产

- Matched Powertrain

- Lube Layout to maintain

Main Technical Parameter

| 主要技术参数 # Bucket motion times 工作时间 |  |
| --- | --- |
| Raising Time: 举臂时间: | 9.6s |
| Lowering Time: 落臂时间: | 5.37s |
| 翻斗时间: Tipping Time: | 2.5s |
| Max. Lifting Height 最大举升时的整机高度： | 5088mm |
| Max. Discharging Height 最大卸载高度： | 1805mm |
| 最大举升时的铰销高度 | 3411mm |
| Min. Discharging Distance: 最小卸载距离： | 1660mm |
| Discharging Angle: 最大卸载角度： | 42° |
| Wheel Base: 轴距： | 3600mm |
| Leaving Angle: 离去角 | 15° |
| Rear Axle Oscillation Angle: 后桥摆动角： | ±8° |
| Min. Off-ground Clearance: 最小离地间隙： | 358mm |
| Max. gradeability: 爬坡能力： | 25% |
| Max. Steering Angle: 最大转向角： | ± 42 ° |

| Dimension (tramming): 外形尺寸（运输状态） | 9800mm×2525mm×2500mm |
| --- | --- |
| Bucket Capacity(SAE 2:1Heaped): 铲斗容量(SAE堆装2:1) | 4m³ |
| Weights: 重量： |  |
| Gross Weight: 设备重量： | 27500 kg |
| Rated Loading Operating Capacity: 额定载重量： | 8000 kg |
| Axle weights without load 空载 |  |
| Front Axle: 前桥载荷 | 12500kg |
| Rear Axle: 后桥载荷 | 15000kg |
| Axle weights with load 满载 |  |
| Front Axle: 前桥载荷 | 26000kg |
| Rear Axle: 后桥载荷 | 11500kg |
| Driving Speeds Forward and Reverse ： 运行能力： |  |
| First Gear: 1档 | 0~4.2km/h |
| Second Gear: 2档 | 0~8.5km/h |
| Third Gear: 3档 | 0~16.5km/h |
| Forth Gear: 4档 | 0~28.0km/h |
| Min. Steering Radius: 最小转向半径： |  |
| Outer / Inner 外侧 / 内侧 | 6600mm/3560mm |

Main Configuration

主要配置

| Diesel Engine 发动机： |  |
| --- | --- |
| Manufacturer: 制造厂： | DEUTZ 德国道依茨 |
| Model: 型号： | BF6M1015C |
| Rated Power: 额定功率 | 223KW@2100 rpm |
| Features: 特点： | Water Cooled, Turbo Charged 水冷, 增压 |
| Hydraulic Torque Converter: 液力变矩器： |  |
| Manufacture: 制造厂： | DANA 美国德纳 |
| Model: 型号： | C5500 Series |
| Power Shift Transmission： 变速箱： |  |
| Manufacture: 制造厂： | DANA 美国德纳 |
| Model: 型号： | R36000 Series |
| Features 特 点: | 前/后四档 电子控制 |
| Drive Axle 驱动桥： |  |
| Manufacture: 制造厂： | DANA 德 纳 |
| Type: 类型： | 前桥固定，后桥摆动±8° |
| Tyres 轮胎： |  |
| Manufacture: 制造厂： | China Tyres Company 中国轮胎 |
| Model: 型号： | 18.00-25 |

| Hydraulic System 液压系统： |  |
| --- | --- |
| Components: 组成： | Servicing system, steering system, braking system, fill oil system, 工作系统、转向系统、 制动系统 、补油系统 |
| Features: 特 点: | Service and steering hydraulic systems are equipped with PARKER Hydraulic Pumps and Valves, and brake hydraulic system equipped with Mico hydraulic components. The oil flow from steering hydraulic pump is directed to bucket hydraulics when steering is not used. Rated Pressure of service hydraulics: 19Mpa 液压泵和阀采用派克品牌，制动系统液压元件采用米肯品牌。 采用双泵合流，减少能量损失。工作系统压力19Mpa |
| Braking System 制动系统 |  |
| Components: 组 成: | Service/Parking/Emergency brake 工作制动、停车制动和紧急制动 |
| Features: 特 点: | These brakes are spring applied, hydraulically released type (SAHR)，with fully enclosed, multiple wet discs that are forced-cooled at each wheel end. 工作制动、停车制动为弹簧制动，液压释放. 制动器为全封闭液压湿式多盘制动器 |

| Electric System 电器系统 |  |
| --- | --- |
| Alternator: 发电机 | 28V 80A |
| Batteries: 蓄电池 | 12VX2 165Ah |
| Starter: 启动马达 | 24V 6.5 KW |
| Lights: 工作灯 | Front:3 pcs Rear:5 pcs |
| Alarm: 报警提示 | Rotation Warning Lamp旋转报警灯 |
| Cab: 驾驶室： |  |
| Features: 特点： | - ROPS/FOPS approved 符合ROPS/FOPS标准 - Side Seated operator for bi-directional operation 驾驶员侧坐以便双向操作 - Adjustable Spring Cushioned Seat with Three Points Belt 可调弹簧减震座椅配安全带 |

| Remote Control System 无线遥控系统 |  |
| --- | --- |
| Vehicular unit 车载单元 | Volt：24VDC Current：＜100mA Wireless Communication Signal Frequency ：2.4GHz； Communication Signal Transmission Speed：1Mbps； Output of digital value, input of analog quantity and digital value expanded as required. |
| Remote Unit 遥控单元 | Volt工作电压：12.8VDC Current工作电流：＜150mA Wireless Communication Signal Frequency无线通信信号频率：2.4GHz； Communication Signal Transmission Speed通信信号传输速率：1Mbps； Battery电池规格：12.8VDC/1.4Ah Lithium iron phosphate battery磷酸铁锂电池 Input of analog quantity and LED display expanded as required. 可根据需要进行拓展模拟输入和 液晶显示等 |
| Communication transmission 通信传输距离 | 40m~180m |

Optional equipment: 可选配置：

Lincoln auto lube with timer: 林肯润滑系统

Ansul Checkfire, autofire suppression: 安索集中灭火系统

Fully Enclosed Cab with Air Conditioner 全封闭空调驾驶室 Customized bucket capacity

可定制其他斗容