Underground Diesel LHD XYDC-10(10T)

Technical specification

The Low Profile Diesel LHD is a low profile which designed to perform in mining operations with limited heights. It can be operated in mine sections with headroom as low as 1.6m.

Characteristics

Comfortable Operation

1. Ergonomics Canopy with generous operation room,

2. Noise ≤85db

3. Adjustable operator’s seat

Efficient Production

- Matched Powertrain

- Digital Monitor & diagnose

- Lube Layout to maintain

Structure

1. Z-bar device

2. Lockup for boom & steering

3.V shape bucket

Safe Operation

1.SAHR brake system

2.ROPS/FOPS Certificate

Main Technical Parameter

| Dimension (tramming): | 9185mm×3200mm×1600mm |
| --- | --- |
| Maximum Width(Cabin): | 3285mm |
| Bucket Capacity(SAE): | 4.6m³ |
| Weights |  |
| Operating Weight: | 27000 kg |
| Rated Loading Capacity: | 10000 kg |
| Axle weights without load |  |
| Front Axle: | 11000kg |
| Rear Axle: | 16000kg |
| Axle weights with load |  |
| Front Axle: | 25000kg |
| Rear Axle: | 12000kg |
| Driving Speeds Forward and Reverse |  |
| First Gear: | 0~4.6km/h |
| Second Gear: | 0~9.6km/h |
| Third Gear: Fourth Gear: | 0~16.1km/h 0~27.2km/h |
| Min. Steering Radius |  |
| Outer / Inner: | 6708mm/3192mm |

Main Configuration

| # Bucket motion times |  |
| --- | --- |
| Raising Time: | 7.2s |
| Lowering Time: | 2.5s |
| Tipping Time: | 2.3s |
| Max. Lifting Height: | 4300mm |
| Max. Discharging Height: | 1960mm |
| Min. Discharging Distance: | 952mm |
| Discharging Angle: | 36° |
| Wheel Base: | 3600mm |
| Leaving Angle: | 12° |
| Rear Axle Oscillation Angle: | ±8° |
| Min. Off-ground Clearance: | 280mm |
| Max. gradeability: | 22% |
| Max. Steering Angle: | ± 42 ° |

| Diesel Engine |  |
| --- | --- |
| Manufacturer: | Mercedes Benz |
| Model: | OM926 |
| Rated Power: | 240KW@2200 rpm |
| Features: | Water Cooled, Turbo Charged |
| Hydraulic Torque Converter |  |
| Manufacture: | DANA |
| Model: | C5500 Series |
| Power Shift Transmission |  |
| Manufacture: | DANA |
| Model: | R36000 Series |
| Features: | F4/R4 Electronic control |
| Drive Axle |  |
| Manufacture: | KESSLER |
| Model: | D101 |
| Tyres |  |
| Manufacture: | China Tyres Company |
| Model: | 20.5-25-TL-L5S |

| Electric System |  |
| --- | --- |
| Alternator: | 28V 100A |
| Batteries: | 12VX2 165Ah |
| Lights: | Front: 4 pcs Rear: 4 pcs |
| Sensors: | Fuel/Oil level sensors, Sensors of service, steering, brake system, Limit Sensor |
| Alarm: | Red/green indicators Rotation Warning Lamp Low oil level warning/Oil pressure warning |
| Monitor Screen: | IFM CR1200 7.0” LED |
| Controller: | IFM CR2532 CPU 50MHz |
| Camera System: | ECCO Camera 3pcs 7.0” LCD 2pcs |
| Cab |  |
| Features: | - ROPS/FOPS approved - Side Seated operator for bi-directional operation - Adjustable Spring Cushioned Seat with Three Points Belt |

| Hydraulic System |  |
| --- | --- |
| Components: | Servicing system, steering system, braking system, fill oil system |
| Features: | Service and steering hydraulic systems are equipped with PARKER Hydraulic Pumps and Valves, and brake hydraulic system equipped with Mico hydraulic components. The oil flow from steering hydraulic pump is directed to bucket hydraulics when steering is not used. |
| Braking System |  |
| Components: | Service/Parking/Emergency brake |
| Features: | These brakes are spring applied, hydraulically released type (SAHR)，with fully enclosed, multiple wet discs that are forced-cooled at each wheel end. |

Optional equipment

Lincoln auto lube with timer

Ansul Checkfire, autofire suppression

Fully Enclosed Cab with Air Conditioner

Customized Bucket Capacity