Underground Remote LHD XYWJY-1.5

Technical specification

This remote LHD is capacity of 3.5 tons. It’s driven by Deutz Original Diesel Engine BF4M1013EC, equipped Visible Remote Control System. This LHD is special for dangerous and narrow underground mine. It improves productivity, and reduces the pollution efficiently.

Characteristics  特点

Structure 结构合理

1. Z-bar device

Z型反六杆工作机构

2. Lockup for boom & steering

大臂及转向均设有安全锁定销

3. V shape bucket

V型铲斗

Efficient Productiom 高效生产

- Matched Powertrain

精确匹配的传动系统

- Lube Layout to maintain

各润滑点布置合理，方便维护

Safe Operatiom 安全的工作环境

SAHR brake system

工作制动、停车制动为弹簧制动，液压释放

Visible Remote Control视距遥控系统

- Operations of Remote mode is same as manual mode.与驾驶室操作相比具有相同的控制装置，便于操作，性能可靠

- Remote and Manual mode shifted freely.

遥控/手动任意切换，互不影响

Main Technical Parameter

| 主要技术参数 Bucket motion times 工作时间 |  |  |
| --- | --- | --- |
| Raising Time 举臂时间 |  | 6.0s |
| Lowering Time 落臂时间 |  | 4.0s |
| Tipping Time 翻斗时间 |  | 3.0s |
| Driving Speeds Forward and Reverse 运行能力 |  |  |
| First Gear 1档: |  | 0~4.4 km/h |
| Second Gear 2档 |  | 0~10.7 km/h |
| Third Gear 3档 |  | 0~19.4 km/h |
| Wheel Base 轴距 |  | 2501mm |
| Leaving Angle 离去角 |  | 15° |
| Max. gradeability 爬坡能力 |  | 25% |
| Rear Axle Oscillation Angle 后桥摆动角 |  | ±8° |
| Min. Off-ground Clearance 最小离地间隙 |  | 286mm |
| Max. Steering Angle 最大转向角 |  | ± 40 ° |
| Min. Steering Radius 最小转向半径 |  |  |
| Outer / Inner 外侧 / 内侧 | 4720mm/2660 mm |  |

| Dimension (tramming) 外形尺寸（运输状态） | 6750mm×1760mm×2080mm |
| --- | --- |
| Bucket Capacity(SAE) 铲斗容量(SAE) | 1.5m³ |
| Weights 重量 |  |
| Operating Weight 操作重量 | 11200 kg |
| RatedLoading Capacity 额定载重量 | 3500 kg |
| Axle weights without load 空载 |  |
| Front Axle 前桥载荷 | 5040 kg |
| Rear Axle 后桥载荷 | 6160 kg |
| Axle weights with load 满载 |  |
| Front Axle 前桥载荷 | 9940 kg |
| Rear Axle 后桥载荷 | 4260 kg |
| Max. tractive force 最大牵引力 | 90KN |
| Breakout force 最大铲取力 | 55KN |
| Max. Lifting Height 最大举升时的整机高度 | 3550mm |
| Max. Discharging Height 最大卸载高度 | 1476mm |
| Bucket pivot at Max. lifting 最大举升时的铰销高度 | 2590mm |
| Min. Discharging Distance 最小卸载距离 | 1076mm |
| Discharging Angle 最大卸载角度 | 45° |

Main Configuration

主要配置

| Diesel Engine 发动机 |  |
| --- | --- |
| Manufacturer 制造厂 | DEUTZ 德国道依茨 |
| Model 型号 | BF4M1013EC |
| Rated Power 额定功率 | 115KW@2300rpm |
| Features 特点 | Water-Cooled, Turbo Charged 水冷, 增压 |
| Hydraulic Torque Converter 液力变矩器 |  |
| Manufacture 制造厂 | DANA 美国德纳 |
| Model 型号 | C272 |
| Power Shift Transmission 变速箱 |  |
| Manufacture 制造厂 | DANA 美国德纳 |
| Model 型号 | RT20326 |
| Features 特 点 | 前/后三档 F3/R3 |
| Drive Axle 驱动桥 |  |
| Manufacture 制造厂 | Xingye Machinery 兴业机械 |
| Model 型号 | XY1.5.4 |
| Tyres 轮胎 |  |
| Manufacture 制造厂 | China Tyres Company 中国轮胎 |
| Model 型号 | 12.00-24 |

| Hydraulic System 液压系统 |  |
| --- | --- |
| Components 组成 | Servicing system, steering system, braking system, fill oil system 工作系统、转向系统、 制动系统 、补油系统 |
| Features 特 点 | 液压系统采用齿轮泵 压力14Mpa |
| Braking System 制动系统 |  |
| Components 组 成 | Service/Parking/Emergency brake 工作制动、停车制动和紧急制动 |
| Features 特 点 | These brakes are spring applied, hydraulically released type (SAHR)，with fully enclosed, multiple wet discs that are forced-cooled at each wheel end. 工作制动、停车制动为弹簧制动，液压释放.制动器为全封闭液压湿式多盘制动器 |

Optional equipment 可选配置

Lincoln auto lube with timer 林肯润滑系统

Ansul Checkfire, autofire suppression 安索集中灭火系统

| Electric System 电器系统 |  |
| --- | --- |
| Alternator 发电机 | 24V 35A |
| Batteries 蓄电池 | 12VX2 100Ah |
| Lights 工作灯 | Front: 2 pcs Rear: 2 pcs |
| Cab 驾驶室 |  |
| Features 特点 | - ROPS/FOPS approved 符合ROPS/FOPS标准 - Side Seated operator for bi-directional operation 驾驶员侧坐以便双向操作 |

Control Board                                                控制面板

Central Articulated

动力机构

Central Articulated                                            中央铰接

Remote Control System遥控系统

- Communication Mode：WIFI-AdHoc / AP

- The service and control funcations are as same as manual mode, it’s reliable and easy to operate.

与驾驶室操作相比具有相同的控制装置，便于操作，性能可靠

- Remote and Manual mode can be shifted freely.

遥控/手动任意切换，互不影响

- Each indicators show operation status of each system.

工作指示：指示灯指示系统工作状态

- The accelerator, Boom up&down, Bucket loading&unloading, Turn left&right are proportional.

比例控制：油门控制、大臂升臂/降臂、铲斗装料/卸料、左转/右转；

- Control Switches: Start&Stop, Emergency, Forward&Reverse, Gear shift, Parking brake, Service brake, head&tail lights, horn, etc.

开关控制：启动、熄火、前进/后退、快/慢档、停车制动（驻车刹车）、行车制动（辅助刹车）、前灯、后灯、喇叭；

- Safe Mode: The emergency brake system will be enabled while Communication failure, vehicle failure, operation failure. 安全模式：通信故障、机车故障、操作错误时启动紧急刹车保护；

- The battery is recharged. 采用可充电电池

Configuration: 主要配置

Remote control unit, Signal reveiver unite, Hydraulic System, Electric unit.

主要配置包括：操作单元、车载单元、液压单元、电气单元

Remote control Unit操作单元

- Voltage: 12.8V DC

- Current: < 95mA

- Wireless communication distance: ＞80m

- Wireless signal frequency: 2.4GHz，IEEE 802.11b/g/n；

- Signals transmission rate: 1Mbps；

- Battery Type: 12.8V DC/1.4Ah Lithium battery

- 20 routes Digital input interface, 8 routes Analogue input interface

Hydraulic System车载单元

- Voltage: 24VDC；

- Current: ＜500mA (No load, it depends on load)

- Wireless communication distance: ＞80m

- Wireless signal frequency: 2.4GHz，IEEE 802.11b/g/n；

- Signals transmission rate: 1Mbps；

- 20 routes on-off control signal output, 8 routes PWM proportional control signal output.

- It can extend digital outputs, analogue or digital inputs.

| 1.5立方铲运机参数对比 |  |  |
| --- | --- | --- |
|  | XYWJY-1.5 | LH203 |
| 额定载重量 kg | 3500 | 3500 |
| 斗容 m³ | 1.5 | 1.5 |
| 操作质量KG | 11200 | 8700 |
| 翻斗铲取力KN | 55 | 55 |
| 发动机 | BF4M1013EC | BF6L914 |
| 功率 | 115KW/2300RPM | 71.5KW/2300RPM |
| 变矩器 | DANA C272 | DANA SOHC2122 |
| 变速箱 | RT20326 | RT20324 |
| 车桥 | XINGYE | DANA 14D1441 |
| 轮胎 | 12.00-24 | 12.00-20 20PLY |
| 长度 mm | 6750 | 7040 |
| 宽度 mm | 1760 | 1480 |
| 高度 mm | 2080 | 1840 |
| 最大卸载高度 mm | 1476 | 1220 |
| 卸载角度 | 45 | 45 |
| 最小卸载距离 mm | 1076 | 1155 |
| 最小离地间隙 mm | 286 | 260 |
| 轴距 mm | 2501 | 2505 |
| 转弯半径 内/外 mm | 2660/4720 | 2740/4685 |
| 转向角度 | 40 | 40 |
| 车桥摆动角度 | 8 | 10 |
| 举升缸径 mm | 125*2 | 180*1 |
| 举升压力Mpa | 14 | 21 |
| 翻斗缸径 mm | 150 | 140 |
| 翻斗压力Mpa | 14 | 21 |
| 转向缸径 mm | 90 | 100 |
| 转向压力Mpa | 14 | 15 |
| 举臂时间 s | 6 | 5.5 |
| 落臂时间 s | 4 | 2.9 |
| 翻斗时间 s | 3 | 3 |
| 最大牵引力KN | 90 | \ |
| 车速 km/h |  |  |
| 1档 | 4.4 | 5 |
| 2档 | 10.7 | 9.3 |
| 3档 | 19.4 | 24 |
| 4档 |  |  |
| 发电机 | 24V*35A | 24V*55A |
| 启动马达 | 4KW | 4KW |
| 蓄电池 | 100Ah*2 | 70Ah*2 |
| 照明灯 | 40w | 65w |
| 遥控系统 | 北京ELINK | 德国HBC |