XYWJ-1.5 Underground Diesel LHD(铲运机)

XYWJ-1.5 Diesel LHD View:

| I. Main Configuration 主要配置 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1. |  | Diesel Engine: 发动机: |  |  |  |  |
|  |  | Manufacture: 制 造 厂 |  | DEUTZ 德国DEUTZ公司 |  |  |
|  |  | Model: 型 号: |  | BF6L912W (Air-cooled) |  |  |
|  |  | Rated Power: 额 定功 率: |  | 63KW@2300RPM |  |  |
|  |  | Optional Engine: 可 选： |  | F5L914(70KW@2300RPM) |  |  |
| 2. |  | Hydraulic Torque Converter: 液力变矩器: |  |  |  |  |
|  |  | Manufacture: 制 造 厂: |  | DANA 美国DANA公司 |  |  |
|  |  | Model: 型 号: |  | C270 Series |  |  |
| 3. |  | Power Shift Transmission: 动力换档变速箱: |  |  |  |  |
|  |  | Manufacture: 制 造 厂: |  | DANA 美国DANA公司 |  |  |
|  |  | Model: 型 号: |  | RT20000 Series |  |  |
|  |  | Type: 类 型: |  | Power Shift 动力换档 |  |  |
|  |  | Gears: 档位数: |  | 3 |  |  |
| 4. |  | Drive Axle: 驱动桥 |  |  |  |  |
|  |  | Manufacture: 制 造 厂: |  | Yantai Xingye Machinery 烟台兴业机械股份有限公司 |  |  |
|  |  | Model: 类 型: |  | Front axle features NO-SPIN differential, the rear axle has self-lubricating and oscillates a total 16° (8° each side). 防滑差速器，后桥带有自润滑摆动架，摆动角±8°。 |  |  |
| 5. |  | Braking System 制动系统 |  |  |  |  |
|  |  | Components: 组 成: |  | SAHR brakes for service operation as well as parking/ emergency. SAHR工作制动 、停车制动、紧急制动 |  |  |
|  |  | Features: 特 点: |  | These brakes are the Spring Applied, Hydraulically Released type, with fully enclosed, multiple wet discs that are force-cooled at each wheel end. 工作制动、停车制动为液压释放，弹簧释放，全封闭式湿式多片摩擦片集成在轮边上。 |  |  |
| 6. |  | Hydraulic System 液 压 系 统: |  |  |  |  |
|  |  | Components: 组 成: |  | Service system, steering system, braking system, charging system 工作系统、转向系统、 制动系统、 补油系统 |  |  |
|  |  | Features: 特 点: |  | The servicing pump, steering valve, multi-directional control valve, conversion valve, are all imported famous brand. Such as the steering pump adopts BOSCH brand, purge hydraulic valve and foot valve adopts Parker SAFIM brand. 主要元件采用进口件（工作泵、多路阀、转向阀、转向泵、充液阀、脚踏阀等。） |  |  |
|  | 7. | Electrical System 电器系统： |  |  |  |  |
|  |  | Power Voltage: 工作电压： |  | 24V |  |  |
|  |  | Electrical Appliance: 电 器： |  | Siemens Brand and Zhengtai Brand 西门子和正泰产品 |  |  |
|  | 8. | Others: 其 它： |  |  |  |  |
|  |  | Tires: 轮胎： |  | Guizhou brand, 12.00/2.5-24 贵州前进 12.00/25-24 |  |  |
|  |  | Hydro-cylinder: 油 缸： |  | Self-made, the grease oil adopts imported combined seal 自制，油封为进口材料组合式密封 |  |  |
|  |  | Blade: 铲 刃： |  | Hardness: HB500, adopt special alloys and anti-wear material. 硬度大于500HB, 采用特种合金耐磨钢 |  |  |
|  | II. |  | Main Technical Date: 主要技术参数： |  |  |  |
|  | 1. |  | Main Dimensions: |  |  |  |
|  |  |  | Dimension (transport): 外形尺寸（运输状态）： |  | 6750mm×1760mm×2080mm |  |
|  | 2. |  | Weights &Axle Loading Weight: 重量和桥载荷： |  |  |  |
|  |  |  | Bucket Capacity (SAE Heaped): 铲斗容积（SAE堆装） |  | 1.5 m³ (2 yard³) |  |
|  |  |  | Rated Loading Capacity: 额定载重量： |  | 3000 KG |  |
|  |  |  | Machine Weight: 整机重量： |  | 11200 KG |  |
|  | 3. |  | Driving Speeds Forward and Reverse ： 运行能力： |  |  |  |
|  |  |  | 1st Gear: 1档： |  | 0~4.4 Km/h |  |
|  |  |  | 2 nd Gear: 2档： |  | 0~10.7 Km/h |  |
|  |  |  | 3 rd Gear: 3档 |  | 0~19.4 Km/h |  |
|  | 5. |  | Distance: 距离： |  |  |  |
|  |  |  | Max. Discharging Height 最大卸载高度： |  | 1476mm |  |
|  |  |  | Min. Discharging Distance: 最小卸载距离： |  | 1076mm |  |
|  | 6. |  | Capacities: 能 力： |  |  |  |
|  |  |  | Max. Climbing Ability: 最大爬坡能力： |  | 15° |  |
|  |  |  | Max. Tractive Force: 最大牵引力: |  | 98 KN |  |
|  |  |  | Max. Shovel Force: 最大铲取力： |  | 92KN |  |
|  | 7. |  | Steering: 转 向： |  |  |  |
|  |  |  | Max. Steering Angle: 最大转向角： |  | ± 40 ° |  |
|  |  |  | Min. Steering Radius: 最小转弯半径： |  | Inner:2660mm 内侧：2660mm | Outer: 4720mm 外侧：4720mm |
|  | III. |  | Scooptram Parts Picture: 整机部分图片： |  |  |  |

Complete Machine                                    Working Part

整机图                                             工作机构

Rear Car                                             Driver’s Cab

后车体                                                  驾驶

Driving Part                                     Central Articulated

动力机构                                             中央铰接

Operation Part                                           Control Board

操纵机构                                                控制面板